AMPHORA — ENGINEERING MEMORANDUM

TO: Engineering

FROM: Jeff, CEO

DATE: July 8, 2026

RE: Engine upgrade plan — prompt-driven envelope synthesis, closed lab-in-the-loop, and manufacturing-release output

# 1. Objective and Scope Shift

The v5 engine today accepts a formal technical envelope (the Jepson envelope was unusually precise) and terminates at a CRO handoff packet. Both boundaries must move:

- Input: future programs arrive as a prompt — verbal brief, e-mail, or one-page ask (“battery cathode for X,” “DLE lithium-selective membrane,” “room-temperature semiconductor-class material”). The engine must synthesize the envelope itself, under governance.

- Output: the terminal artifact is not a validated candidate but a manufacturing-ready material: locked formulation/composition + process window + QC/SPC release criteria + supplier-qualified BOM, transferable to a toll producer or internal line.

Everything below preserves the v5 non-negotiable: auto-generated content enters at low authority tiers, passes the source ledger, and fails closed. We extend the pipeline; we do not relax it.

# 2. Target Pipeline (end state)

prompt → envelope-synthesis → domain-pack → generate/score/gate (existing core) → active-learning DOE → evidence-intake → model promotion → pilot scale-up → process-qc-twin release → manufacturing package

# 3. Workstream A — Envelope Synthesis Service (prompt → spec)

New service, same contract style as the split repos. Consumes an unstructured brief; emits a machine-readable ProductEnvelope (generalization of the current envelope; versioned schema alongside CandidateEnvelope).

- Elicitation pass. LLM-driven interrogation of the brief: intended service environment, performance targets, regulatory jurisdiction, cost ceiling, volume, form factor. Unanswered items become an explicit ambiguity register, not silent defaults.

- Constraint formalization. Each requirement becomes a typed constraint: hard gate, soft objective, or info row — with units, test-method anchor (ASTM/IEC/ISO), and a provenance tag: CUSTOMER_STATED / INFERRED / DEFAULTED.

- Authority discipline. INFERRED and DEFAULTED constraints enter at low authority and are flagged in every downstream report until a human (customer or PM) countersigns. Envelope sign-off is a blocking gate before any candidate generation spends compute.

- Round-trip test. Acceptance: feeding the Jepson envelope's prose back through the service must reproduce the current envelope constraints (VS 65±2%, VOC ≤340, 1:1, DFT 4.5–6.0, etc.) with zero silent additions. That is the regression baseline.

# 4. Workstream B — Domain-Pack Generator

A domain pack = representation adapter + property calculators/simulators + materials library + design rules + test-method map + evidence schema. The coatings pack is hand-built; industrialize the pattern:

- Representation adapters. Abstract the molecule layer behind an interface: SMILES/RDKit (organics, membranes, polymers); composition + crystal structure via pymatgen/CIF (battery electrodes, semiconductor-class inorganics); polymer/membrane descriptors (MWCO, charge density, free volume) for DLE. Candidate identity, hashing, and provenance must be representation-agnostic.

- Calculator/simulator plugin API. Same ScoreEnvelope contract; declared authority tier per plugin. Closed-form arithmetic (stoichiometry, mass/charge balance, theoretical capacity mAh/g, membrane flux equations) = REAL_ARITHMETIC. DFT/MD/CALPHAD wrappers = MODELED_DIRECTIONAL until benchmarked against known-answer sets. Untrained ML = excluded from objectives, period.

- Library builder. Semi-automated ingestion of cited constants (PubChem, Materials Project, ICSD, supplier TDS) through the existing source-ledger rejection path. No uncited constant loads — same rule as today.

- Known-answer benchmarks per domain. Before a pack goes live it must re-derive an established material (as coatings re-derived MIL-DTL-24441 chemistry): e.g., NMC/LFP for cathodes, a commercial NF membrane for DLE. Failure blocks the pack.

- Test-method mapper. Auto-draft the qualification matrix from the envelope: coin/pouch cycling + EIS + rate maps (batteries); flux/rejection/Li-Mg selectivity + fouling + burst (DLE); Hall mobility/band gap/wafer-level metrology (semiconductor-class). Output feeds the CRO packager unchanged.

# 5. Workstream C — Close the Lab Loop (highest priority, already scoped)

- Execute the first CRO batch (V0/V3/V4 + Envelope-65/Enhanced-65/Lineage-60 with controls). Nothing below matters until data exists.

- Harden evidence-intake-validator as the sole write path for external data: raw files, calibration, custody, controls, deviation records. Add automated parsing for instrument exports (KU, EIS spectra, B117 ratings) to cut intake latency.

- Activate the GP optimizer at ≥5 measured points per objective; implement batch active learning (qEI or similar) so each CRO round is selected for information gain, not convenience.

- Model promotion pipeline. Train estimated-tier models (pot-life/KU, Tg/viscosity) on measured data; auto-issue model cards; promotion/demotion of authority tiers is computed from held-out error against pre-registered bars — never hand-edited.

- Red-team v2. Move from claim-linting to substance attacks: hostile-null baselines every proxy must beat, adversarial candidates that game single-metric scoring, circularity detection on spec-derived fitness, and a CI invariant that artifact score cannot rise while the evidence score is flat.

# 6. Workstream D — Manufacturing Release Layer

The current process-qc-twin is ROADMAP-tier. Build it out as the terminal gate:

- Process model. Encode the bench process (grind sequence, tip speed, Hegman endpoints, letdown) as a parameterized recipe; add scale-up correlations (tip-speed/shear scaling, heat load, batch-time) so pilot instructions are generated, not rewritten.

- SPC and release criteria. Per-batch gates from engine theory: density ±0.02 g/mL, NV ±1%, fineness, KU window; control charts seeded from theoretical values and tightened from measured batches. Release = all gates green + retained sample logged.

- Supplier qualification chain. Lot CoA ingestion recomputes stoichiometry per batch (AHEW drift is the known killer); dual-source rules and declaration status (PFAS/TSCA/TAA) block release automatically if stale.

- Tech-transfer package generator. One command emits: locked formulation record, process instructions, QC plan, SDS-linked EHS sheet, and the evidence dossier hash-chain. Definition of done for any program = this package accepted by a manufacturing partner.

# 7. Sequencing and Dependencies

| Order | Workstream | Depends on | Acceptance test |
| --- | --- | --- | --- |
| 1 (now) | C1–C2: CRO batch + intake hardening | Supplier declarations | First accepted external dataset in the ledger |
| 2 | C3–C5: optimizer, model promotion, red-team v2 | C1 data | Round-2 slate chosen by optimizer; one proxy promoted or killed on data |
| 3 (parallel) | A: envelope synthesis | None (independent) | Jepson round-trip regression passes; ambiguity register demo on a verbal brief |
| 4 | B: domain-pack generator | A (envelope schema) | One new pack (recommend DLE membranes: polymer chemistry, near-term testable) re-derives a known material and emits a CRO-ready campaign |
| 5 | D: manufacturing layer | C loop ≥2 rounds | Tech-transfer package for AMP-EP-0001-class primer accepted by a toll coater |

Domain-pack pilot order rationale: DLE membranes first (polymer domain, reuses RDKit adapter, cheap flux/selectivity bench tests, fast loops). Batteries second (new inorganic representation, coin-cell validation is slow but standardized). Semiconductor-class materials last — highest simulation cost, longest validation, and physically the most speculative; treat as discovery-annex work, not a committed product lane.

# 8. Engineering Ground Rules

- Every new service speaks CandidateEnvelope/ScoreEnvelope (or the versioned ProductEnvelope superset). No side channels.

- Auto-generated envelopes, rules, and constants are born low-authority and are promoted only by data or human countersignature. The generator lives inside the governance system, not above it.

- The optimizer inclusion rule is unchanged: SCORED + usable_by_optimizer + OPTIMIZATION_OBJECTIVE + permitted tier. New domains get no exemptions.

- Manufacturing claims (“procurement-ready,” “qualified,” “release”) stay locked behind process-qc-twin evidence, exactly as performance claims are locked behind CRO evidence today.

Bottom line: two new ends on the same spine. A front end that turns a prompt into a signed, governed envelope; a back end that turns validated candidates into a transferable manufacturing package. The middle — generate, score, gate, learn — is already built and already honest. Keep it that way.